🛰️

GPS SaaS Platform

Complete User Manual

Version 1.0  |  All Roles Guide

Three Roles. One Platform. ADMIN ¦ CLIENT ¦ SUBUSER

Real-time vehicle tracking • Geofencing • Alerts • Analytics • Billing

# 1. Platform Overview

GPS SaaS is a multi-tenant, cloud-hosted GPS tracking platform that lets businesses track their entire vehicle fleet in real time. It supports GT06-compatible GPS devices, provides live maps, geofence alerts, driver scoring, billing management, and full white-label branding.

## 1.1  How the Platform Works

| Component | What It Does |
| --- | --- |
| GPS Device | Your vehicle tracker (GT06 protocol) connects to our TCP server on port 5000 |
| TCP Server | Receives raw GPS packets from devices, decodes them, and queues them |
| Backend API | Processes data, stores to database, checks geofences, fires alerts |
| WebSocket | Pushes live location updates to your browser in real time |
| Frontend App | Web app at your domain — view map, devices, alerts, billing |
| Admin Panel | Separate portal for administrators at admin.yourdomain.com |

## 1.2  Three User Roles

| Role | Access Level | Typical User |
| --- | --- | --- |
| ADMIN | Full system access | System owners / operators |
| CLIENT | Manage own fleet | Business customers |
| SUBUSER | View only | Drivers / dispatchers |

# 2. Getting Started

## 2.1  System Requirements

- Web browser: Chrome 90+, Firefox 88+, Safari 14+, Edge 90+

- Internet connection: minimum 1 Mbps

- Screen resolution: 1280×720 minimum (1920×1080 recommended)

- For GPS device connectivity: TCP port 5000 must be reachable from device SIM

## 2.2  Accessing the Platform

| Portal | URL |
| --- | --- |
| Client Web App | https://yourdomain.com |
| Admin Panel | https://admin.yourdomain.com |
| API Base URL | https://yourdomain.com/api |
| GPS TCP Port | yourdomain.com:5000 |
| Grafana Metrics | https://yourdomain.com:3001 |

## 2.3  First-Time Login (Admin)

After deployment, use these default credentials to log in to the Admin Panel. Change the password immediately after first login.

| Field | Default Value |
| --- | --- |
| Email | admin@yourdomain.com |
| Password | Admin@123 (CHANGE IMMEDIATELY) |

SECTION 3 — ADMIN ROLE

# 3. Administrator Guide

The ADMIN is the system owner. They have complete access to all users, all devices across all tenants, billing plans, and system-wide configuration. The Admin Panel is accessed at admin.yourdomain.com.

## 3.1  Admin Dashboard

The Admin Dashboard shows a live overview of the entire platform.

- Total registered users — all CLIENT accounts

- Total devices — across all clients

- Active subscriptions — currently paying clients

- Live map — showing all online devices from all clients

- System health — backend, database, and Redis status

## 3.2  User Management

Navigate to Admin → Users to manage all registered client accounts.

### 3.2.1  Viewing Users

- See all registered users with name, email, role, and subscription status

- Filter users by status (Active / Inactive)

- View which plan each client is subscribed to

### 3.2.2  Enable / Disable Accounts

Click Enable or Disable next to any user to activate or deactivate their account. Disabled accounts cannot log in and their devices stop showing on any map.

## 3.3  Device Management (Admin View)

Admins can see ALL devices registered across ALL clients. Go to Admin → Devices.

- View every registered device: IMEI, vehicle number, client owner, status

- Filter by online / offline

- See last seen timestamp for each device

## 3.4  Plan Management

Navigate to Admin → Billing to manage subscription plans.

| Plan | Details |
| --- | --- |
| Basic | ₹499/month — up to 5 devices |
| Standard | ₹999/month — up to 20 devices |
| Enterprise | ₹2,499/month — up to 100 devices |

## 3.5  System Monitoring

Grafana is available at https://yourdomain.com:3001 for real-time metrics.

- API request rate — requests per second

- Active TCP connections — devices currently connected

- GPS packets processed — data throughput

- Error rates and response times

- Database and Redis health metrics

## 3.6  Alerts Management (Admin)

Admins can view all alerts across all tenants. Go to Admin → Alerts.

- See all geofence, overspeed, SOS, and system alerts

- Filter by severity: INFO, WARNING, CRITICAL

- View alert location on map (latitude/longitude recorded)

✅ ADMIN: DO's

- Change the default admin password immediately after first login

- Create strong, unique passwords (minimum 12 characters with symbols)

- Regularly check the health dashboard and Grafana metrics

- Monitor active subscriptions and follow up with expiring clients

- Back up the database daily using the provided backup.sh script

- Keep your SSL certificate renewed (auto-renew with certbot recommended)

- Review audit logs weekly for suspicious activity

- Set up CORS_ORIGIN to your exact domain — never use wildcard '*' in production

- Test the /health endpoint regularly to verify all services are running

- Disable unused client accounts promptly when clients leave

- Monitor disk space — GPS history data grows rapidly

- Keep Docker images updated monthly for security patches

✗ ADMIN: DON'Ts

- Never share the admin login credentials with anyone

- Never set JWT_SECRET or JWT_REFRESH_SECRET to simple strings like 'secret' or 'password'

- Never expose port 3001 (Grafana) to the public internet without authentication

- Never delete plans that have active subscriptions linked to them

- Never run docker compose down with --volumes flag — this deletes all data

- Never edit the database directly with SQL unless you understand the schema

- Never leave POSTGRES_PASSWORD, RAZORPAY_SECRET blank in .env.production

- Never commit your .env.production file to GitHub — it contains all secrets

- Never open unnecessary ports in the Lightsail firewall

- Never ignore CRITICAL severity alerts in Grafana

SECTION 4 — CLIENT ROLE

# 4. Client Guide

A CLIENT is a business customer who has subscribed to the GPS SaaS platform. They manage their own fleet of vehicles and devices. The client accesses the platform at https://yourdomain.com.

## 4.1  Account Registration

Clients self-register on the login page by clicking 'Sign Up'. After registration, they can subscribe to a plan and start adding devices.

| Field | Description |
| --- | --- |
| Name | Full name of the business owner or administrator |
| Email | Business email address — used for login and notifications |
| Password | Minimum 8 characters — use a strong password with numbers and symbols |

## 4.2  Subscribing to a Plan

After registration, navigate to Billing to choose and subscribe to a plan.

- Go to Billing from the top navigation

- Select a plan (Basic / Standard / Enterprise)

- Click Subscribe — a Razorpay payment popup will appear

- Complete payment with UPI, card, or netbanking

- Your subscription activates within 1-2 minutes

ℹ️ Payment is processed securely by Razorpay. GPS SaaS never stores your card details.

## 4.3  Adding Devices

Navigate to Dashboard and use the Add Device form. Each device needs:

| Field | Description |
| --- | --- |
| IMEI | 15-digit unique ID printed on the GPS device — must match exactly |
| Vehicle Number | Registration number e.g. MH12AB1234 |
| Model | GPS device model e.g. GT06N, GT06E (optional) |
| SIM Number | SIM card number inside the device (optional but recommended) |

ℹ️ Each IMEI can only be registered once across the entire platform. If registration fails, the IMEI may already be in use.

## 4.4  Configuring Your GPS Device

Once you have added a device in the platform, configure the GPS device to connect to your server. Most GT06 devices are configured via SMS commands:

| Action | SMS Command |
| --- | --- |
| Set Server IP | Send SMS: SERVER,1,yourdomain.com,5000,0# to the SIM in the device |
| Set APN | Send SMS: APN,YOUR_APN# (ask your SIM provider for APN) |
| Set Heartbeat | Send SMS: TIMER,30# (report every 30 seconds) |
| Check Status | Send SMS: STATUS# to get current device status |
| Reboot Device | Send SMS: REBOOT# to restart the device |

ℹ️ Replace 'yourdomain.com' with your actual domain or server IP. Port must be 5000.

## 4.5  Live Tracking

Go to Live Tracking in the navigation menu to see your vehicles on the map in real time.

- The map automatically shows all your online vehicles

- Click any marker to see vehicle details: speed, last seen, vehicle number

- The map updates automatically every time a GPS packet is received (typically every 10-60 seconds)

- Vehicles that have not reported in 5 minutes show as offline

## 4.6  Managing Geofences

Geofences are virtual boundaries on the map. When a vehicle enters a geofence zone, you receive an alert.

To create a geofence:

- Go to Geofences in the navigation

- Enter a name for the geofence (e.g. 'Head Office', 'Warehouse A')

- Select type: Circle (enter centre latitude/longitude + radius in meters)

- Click Add — the geofence is immediately active

To find latitude/longitude of a location: open Google Maps, right-click on the location, and the coordinates appear in the menu.

## 4.7  Understanding Alerts

Alerts are generated automatically when events occur. Go to Alerts in the navigation.

| Alert Type | Severity | Description |
| --- | --- | --- |
| geofence_entry | INFO | A vehicle entered one of your geofence zones |
| overspeed | WARNING | A vehicle exceeded the speed limit (default: 100 km/h) |
| sos | CRITICAL | The SOS button was pressed on the device |
| power_cut | CRITICAL | Main power to the device was disconnected |
| low_battery | WARNING | Device battery level is low |

## 4.8  Dashboard Overview

- Total Devices — all registered devices in your account

- Online — devices currently sending GPS data

- Offline — devices not seen in the last 5 minutes

- Device table — IMEI, vehicle number, status, and last seen time for each device

✅ CLIENT: DO's

- Use a strong password with at least 12 characters including numbers and symbols

- Keep your subscription active — devices stop appearing if subscription expires

- Test GPS device connectivity after first setup by checking Live Tracking

- Create meaningful geofence names (e.g. 'Depot Mumbai', 'Client Site Pune')

- Mark alerts as Read after reviewing them to keep your alert list manageable

- Add SIM number when registering a device — helps with troubleshooting

- Check Alerts page daily to stay informed about fleet events

- Contact support if a device shows as offline for more than 30 minutes

- Renew your subscription before it expires to avoid any tracking interruption

- Keep your registered email accessible — it is used for billing notifications

✗ CLIENT: DON'Ts

- Never share your login credentials with drivers or other staff — create SUBUSER accounts instead

- Never register the same IMEI under multiple accounts — it will cause tracking conflicts

- Never ignore CRITICAL alerts (SOS, power_cut) — they require immediate attention

- Never configure the GPS device to connect to port 80 or 443 — use port 5000 only

- Never delete a device and re-add it to 'fix' issues — contact support first

- Never share your API access token if you are using the API directly

- Never expect immediate tracking for a newly added device — allow 5-10 minutes for first connection

- Never use the same email to register multiple CLIENT accounts

SECTION 5 — SUBUSER ROLE

# 5. Subuser Guide

A SUBUSER is a view-only user created by a CLIENT. They can track vehicles and view alerts but cannot add or modify devices, manage geofences, or access billing. Typical subusers are fleet managers, dispatchers, and supervisors.

ℹ️ SUBUSERs are currently registered the same way as CLIENTs. The ADMIN assigns the SUBUSER role. Contact your administrator to set up your account.

## 5.1  What Subusers Can See

| Feature | Access |
| --- | --- |
| Dashboard | Live overview of all vehicles — status, last seen |
| Live Tracking | Real-time map with all vehicle locations |
| Alerts | All alerts generated for the fleet — can mark as read |

## 5.2  What Subusers Cannot Do

- Add, edit, or delete devices

- Create or manage geofences

- Access billing or subscription information

- View or manage other users

- Send commands to GPS devices

- Access admin panel

## 5.3  Using Live Tracking

Navigate to Live Tracking to view all vehicles on the map.

- Each vehicle shows as a pin on the map

- Click a pin to see: vehicle number, current speed, last update time

- The map refreshes automatically — no manual refresh needed

- Offline vehicles (not seen in 5 minutes) may appear greyed out

✅ SUBUSER: DO's

- Refresh the page if the map stops updating for more than 2 minutes

- Report any CRITICAL alerts immediately to your fleet manager/CLIENT

- Mark alerts as Read after you have reviewed and actioned them

- Use Live Tracking to monitor vehicles during active shift hours

- Contact your administrator (CLIENT) if you believe a vehicle is offline unexpectedly

- Keep your password secure — do not share it with anyone

✗ SUBUSER: DON'Ts

- Do not attempt to access the admin panel or billing sections

- Do not share your login credentials with drivers

- Do not ignore CRITICAL alerts — escalate to your manager immediately

- Do not use the platform on unsecured public Wi-Fi without a VPN

- Do not leave the platform open on a shared/public computer

- Do not contact Razorpay or billing support — only CLIENT/ADMIN handles billing

# 6. GPS Device Configuration

## 6.1  Supported Protocols

The platform supports the GT06 protocol family — compatible with a wide range of Chinese GPS trackers including:

- GT06, GT06N, GT06E — most common

- Concox GT06 series

- JM-VL01, JM-LL01 series

- Any third-party device that implements the GT06 binary TCP protocol

## 6.2  Server Configuration via SMS

| Action | SMS Command |
| --- | --- |
| Set server (format 1) | SERVER,1,DOMAIN,5000,0# |
| Set server (format 2) | IP,DOMAIN,5000# |
| Set APN (Airtel) | APN,airtelgprs.com# |
| Set APN (Jio) | APN,jionet# |
| Set APN (Vi) | APN,portalnmms# |
| Set reporting interval | TIMER,30# (30 seconds) |
| Check connectivity | STATUS# |
| Factory reset | RESET# (use carefully) |

ℹ️ Replace DOMAIN with your actual server domain or IP address. All SMS commands go to the SIM number inside the GPS device.

## 6.3  Troubleshooting Device Connection

| Symptom | Likely Cause | Solution |
| --- | --- | --- |
| Device shows offline | Wrong server IP or port | Re-send SERVER SMS command with correct domain and port 5000 |
| Never connected | Wrong APN setting | Contact SIM provider for correct APN, re-send APN command |
| Intermittent connection | Weak GSM signal | Check signal strength in device area, consider different SIM |
| Wrong location data | GPS lock not acquired | Place device with clear sky view for 5-10 minutes |
| IMEI not recognized | IMEI not registered in platform | Add the device in Dashboard → Devices first |

# 7. Feature Reference

## 7.1  Real-Time Tracking

- Updates every time the device sends a packet (configurable via TIMER SMS)

- Location stored in database for historical playback

- Speed, heading (direction), and timestamp recorded for each point

- Automatically marks device as offline if no data for 5 minutes

## 7.2  Geofencing

- Circle geofences — define a centre point and radius in meters

- An alert is generated each time a vehicle enters the zone

- Multiple geofences can be active simultaneously

- Real-time alert pushed to browser via WebSocket when triggered

## 7.3  Alerts System

| Type | Severity | Trigger |
| --- | --- | --- |
| geofence_entry | INFO | Vehicle crossed into a geofence boundary |
| overspeed | WARNING | Vehicle speed exceeded configured threshold |
| sos | CRITICAL | SOS button activated on device |
| power_cut | CRITICAL | External power disconnected from device |
| vibration | WARNING | Vibration/movement detected (when parked) |
| low_battery | WARNING | Device internal battery is low |

## 7.4  Driver Scoring

Every GPS packet is analysed for driver behaviour. A score from 0-100 is calculated based on:

| Behaviour | Impact |
| --- | --- |
| Harsh Braking | -2 points per event (sudden deceleration > 25 km/h per interval) |
| Harsh Acceleration | -2 points per event (sudden acceleration > 25 km/h per interval) |
| Overspeed | -3 points per event (speed > 100 km/h, configurable) |
| Starting Score | 100 points — deductions applied for each event |

## 7.5  Billing & Subscriptions

- Payments processed via Razorpay — UPI, cards, netbanking supported

- Subscriptions auto-expire on the end date — no auto-renewal

- Upon expiry, device data stops showing but historical data is retained

- CLIENT must manually renew by selecting a new plan and paying

- Admin can see all active subscriptions and their expiry dates

## 7.6  White-Label Branding

Clients with business requirements can customise the platform branding. Navigate to Settings → Branding to set:

- Company name and logo URL

- Primary and secondary brand colours (hex codes)

- Custom domain for white-label deployment

- Support email address shown to end users

# 8. AWS Lightsail Deployment Guide

This section is for the system operator or technical team responsible for deploying and maintaining the GPS SaaS platform on AWS Lightsail.

## 8.1  Infrastructure Overview

| Component | Technology |
| --- | --- |
| Web Server | AWS Lightsail Ubuntu 22.04 (minimum 4GB RAM recommended) |
| Database | PostgreSQL 15 with PostGIS extension (Docker container) |
| Cache / Queue | Redis 7 (Docker container) |
| Reverse Proxy | Nginx (Docker container) — handles SSL and routing |
| Monitoring | Prometheus + Grafana (Docker containers) |
| SSL | Let's Encrypt free certificate (auto-renewal via certbot) |
| CI/CD | GitHub Actions — auto-deploys on push to main branch |

## 8.2  Lightsail Firewall Ports

| Port | Purpose |
| --- | --- |
| 22 | SSH — server access |
| 80 | HTTP — redirects to HTTPS |
| 443 | HTTPS — main web traffic |
| 5000 | GPS TCP — device connections |
| 3001 | Grafana — monitoring (restrict to admin IPs if possible) |

## 8.3  Step-by-Step Deployment

- Create Lightsail instance: Ubuntu 22.04, 4GB RAM, 80GB SSD

- Open firewall ports: 22, 80, 443, 5000, 3001 in Lightsail Console → Networking

- SSH into server and run: scripts/lightsail-setup.sh

- Edit .env.production with all required secrets and your domain

- Point domain DNS A record to your Lightsail static IP

- Get SSL certificate with certbot and copy to nginx/ssl/

- Run: docker compose up -d

- Verify: docker compose ps (all containers should be 'running')

- Add GitHub Secrets and push to main — CI/CD will deploy automatically

- Set up Razorpay webhook pointing to https://yourdomain.com/api/billing/webhook

- Set up daily cron job for backup.sh

## 8.4  Environment Variables Reference

| Variable | Purpose | Notes |
| --- | --- | --- |
| POSTGRES_PASSWORD | Database password | REQUIRED — min 16 chars, random |
| JWT_SECRET | Access token signing key | REQUIRED — 64 char random hex |
| JWT_REFRESH_SECRET | Refresh token signing key | REQUIRED — different 64 char hex |
| RAZORPAY_KEY | Razorpay public key | REQUIRED for billing |
| RAZORPAY_SECRET | Razorpay secret key | REQUIRED for billing |
| RAZORPAY_WEBHOOK_SECRET | Webhook signature verification | REQUIRED for billing |
| CORS_ORIGIN | Allowed frontend origin | Set to https://yourdomain.com |
| OVERSPEED_LIMIT | Speed alert threshold in km/h | Default: 100 |
| LOG_LEVEL | Logging verbosity | info (production), debug (dev) |
| GRAFANA_PASSWORD | Grafana admin password | REQUIRED — change from default |

# 9. API Quick Reference

All API endpoints are at https://yourdomain.com/api. Use Bearer token authentication in the Authorization header.

## Authentication

| Endpoint | Action | Access |
| --- | --- | --- |
| POST /auth/register | Register new user | Public |
| POST /auth/login | Login — returns accessToken | Public |
| POST /auth/refresh | Refresh access token | Public |
| GET /auth/me | Get current user profile | Authenticated |

## Devices

| Endpoint | Action | Access |
| --- | --- | --- |
| GET /devices | List your devices (with live GPS) | CLIENT |
| POST /devices | Register a new device | CLIENT + Active Sub |
| GET /devices/:id | Get single device | CLIENT |
| GET /devices/:id/history | Get GPS history (use ?from=&to=) | CLIENT |
| PUT /devices/:id | Update device info | CLIENT |
| DELETE /devices/:id | Remove a device | CLIENT |

## Billing

| Endpoint | Action | Access |
| --- | --- | --- |
| GET /billing/plans | List all plans | Public |
| POST /billing/create-order | Create Razorpay order | CLIENT |
| GET /billing/subscription | Get my subscription | CLIENT |
| POST /billing/webhook | Razorpay webhook (Razorpay only) | System |

## Alerts, Geofences, Analytics

| Endpoint | Action | Access |
| --- | --- | --- |
| GET /alerts | List all alerts | CLIENT/SUBUSER |
| PUT /alerts/:id/read | Mark alert as read | CLIENT/SUBUSER |
| GET /geofences | List geofences | CLIENT |
| POST /geofences | Create geofence | CLIENT |
| DELETE /geofences/:id | Delete geofence | CLIENT |
| GET /analytics | Get analytics data | CLIENT |

# 10. Troubleshooting Guide

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| Device never shows online | Wrong server IP/port in device | Re-send SERVER SMS. Verify port 5000 is open in Lightsail firewall |
| Device was online, now offline | GSM signal lost or SIM expired | Check SIM balance and signal. Device auto-reconnects when signal returns |
| Payment not activating sub | Webhook not configured | Check Razorpay webhook URL in dashboard. Check backend logs for webhook errors |
| Login says Invalid credentials | Wrong email/password or inactive | Use correct credentials. Contact admin if account is disabled |
| Map shows no vehicles | No active subscription | Go to Billing and subscribe to a plan |
| Alerts not appearing | Device offline or geofence issue | Verify device is online. Check geofence coordinates are correct |
| API returns 401 | Token expired | Call POST /auth/refresh with your refreshToken to get new accessToken |
| Docker container not starting | Missing env variable or port conflict | Check docker compose logs SERVICE_NAME for specific error |
| Database connection failed | Postgres not ready yet | Wait 30s after docker compose up. Check postgres healthcheck status |
| SSL certificate expired | Auto-renewal not set up | Run: sudo certbot renew, then copy new certs to nginx/ssl/ |

# 11. Support & Contact

For technical support, billing questions, or feature requests:

| Issue Type | How to Resolve |
| --- | --- |
| Technical Issues | Check /health endpoint first, then review docker compose logs |
| Billing Queries | Contact your platform administrator (ADMIN) |
| GPS Device Issues | Re-configure using SMS commands in Section 6.2 |
| Password Reset | Contact your administrator to reset your account |

GPS SaaS Platform — User Manual Version 1.0 ¦ All rights reserved